- Automixer Protocol Description

- SDXI-U2 (Serial ASCII Data Exchange Interface)

- Version 1.16

Date: 22.06.2016

copyright © Stemin GmbH, 2009 – 2013. All Rights Reserved.

Document version

| version | date | Name | Remark |
| --- | --- | --- | --- |
| 1.16 | 10.04.2013 | R. Wuppinger | - fixes and corrections in the manual text - changes supporting the new functions of the Automixer devices - document renamed from “SDXI U2 Protocol Description Vx.yy.doc” to “SDXI U2 Automixer Protocol Description Vx.yy.doc - removed “Virtual Talk Station” commands - removed “Request Device Status” commands |

Contents

EMPTY PAGE	5

1	General description	6

1.1	Protocol SDXI	6

1.2	Protocol Extension SDXI-U2 (ASCII Mode)	6

1.3	Serial Parameters	6

1.4	Availability of Telegrams	6

1.5	Important: Enabling Remote Control first	6

1.6	Telegram Specification	7

2	Commands	8

2.1	Definition of General Command Parameter	8

2.2	Version Commands	11

2.2.1	Software Version („U2DM<d>SW?<CR>“)	11

2.2.2	Hardware Version („U2DM<d>HW?<CR>“)	11

2.3	Device Configuration	12

2.3.1	Set Baudrate („U2DM<d>COM<c>BD=<bd>“)	12

2.3.2	Set Remote Interface („U2DM<d>DRI=<s>“)	12

2.4	Encoder Commands	13

2.4.1	Encoder Request Value („U2DM<d>EN<e>?<CR>“)	13

2.4.2	Encoder Rotate Left („U2DM<d>EN<e>RL<CR>“)	13

2.4.3	Encoder Rotate Right („U2DM<d>EN<e>RR<CR>“)	14

2.4.4	Encoder Set Value („U2DM<d>EN<e>=<ev><CR>“)	14

2.5	Status Commands	15

2.5.1	Request Channel Status („U2DM<d>CS<c>?<CR>“)	15

2.5.2	Set Channel Status Remote Update Time („U2DM<d>CS<c>=<t><CR>“)	15

2.5.3	Channel Status Set Mute („U2DM<d>EN<c>MU=<mut><CR>“)	16

# This page is empty.

# General description

## Protocol SDXI

The SDXI-U2 (Serial Data Exchange Interface) should allow interacting, controlling and maintaining a variety of different devices. The serial interface was designed without any higher protocol function like a handshake or something equal. This interface allows the communication of embedded devices with a PC and/or among each other. Every device, a PC or the embedded device is responsible for the correct receiving and interpreting of the telegrams. This refers also to the timing-behaviour, this means that the devices must be able to react and process to the telegrams during runtime, without validating the timing conditions, which could lead to the loss of telegrams. A loss of a telegram can also cause a loss of data because for some telegram a repetition of the telegram is not possible. If the external device is fast enough and is controlled by an interrupt driven interface there should be no problem to handle the telegrams.

## Protocol Extension SDXI-U2 (ASCII Mode)

The SDXI protocol includes a second, higher level protocol interface called SDXI-U2 protocol, which allows the user to communicate and control the device with a simple terminal program using an ASCII-based interface language.

This SDXI-U2 protocol provides a subset of the regular SDXI telegrams respectively SDXI commands, to make the basic functions available for an easy access via various media control devices like AMX, Crestron or something similar.

## Serial Parameters

To select a user specific baud rate, you can select a different baud rate using the according SDXI-U2 telegram. For detailed description, please refer to the manual.

Default parameters:

BAUDRATE:	9600
PARITY: 	NONE 	(fixed)
STOPBIT:	1 	(fixed)
DATABITS:	8 	(fixed)

## Availability of Telegrams

The availability of each telegram depends on optional and device specific software modules as well.

## Important: Enabling Remote Control first

To enable the remote control functionality, it is first necessary to enable the remote interface of the device by sending a “Set Remote Interface” command to the device!

## Telegram Specification

The following table shows the structure of the SDXI-U2 telegrams:

| Byte Index | Byte Quantity | Meaning | value / example |
| --- | --- | --- | --- |
| 0 | 1 | start byte | ‘U’ (0x55H): Start of Telegram |
| 1 | 1 | telegram version | ‘2’ (0x32H): signals that a SDXI-U2 telegram follows (ASCII mode) |
| 2 | 2 … 3 | Device Code | “DM” (Digital Mixer) |
| variable | 1 … 3 | Device Address | “1”, …,“999” |
| variable | 2 … 3 | Object Code | “EN”: Encoder “COM”: ComPort |
| variable | 1 … 3 | Object Address | “0”, …,“255” |
| variable | 2 … 3 | Command Code / Object Property | Command Codes: “RR”: encoder turn right “RL”: encoder turn left “MU”: mute channel “BD”: Baudrate |
| variable | 1 | Operator | ‘=’ set operator ‘?’ request operator |
| variable | 1 … 6 | Parameter 1 (optional) | “-32767”…”32767” |
| variable | 1 | Seperator (optional) | ‘,’ |
| variable | 1 … 6 | Parameter 2 (optional) | “-32767”…”32767” |
| variable | 1 | Seperator (optional) | ‘,’ |
| variable | 1 … 6 | Parameter 3 (optional) | “-32767”…”32767” |
| variable | 1 | Seperator (optional) | ‘,’ |
| variable | 1 … 6 | Parameter 4 (optional) | “-32767”…”32767” |
| variabel | 1 | end byte | 0x0d (HEX) End of Telegram CR |

Example:

| ASCII | “U2DM1EN2=35<CR>” | sets encoder 2 to value 35 (level) |
| --- | --- | --- |
| HEX | 55 32 44 4d 31 45 4e 32 3d 33 35 0d |  |

# Commands

## Definition of General Command Parameter

| Parameter | Symbol | Range | Description |
| --- | --- | --- | --- |
| transmission to the device |  |  | data will be send/transmitted to the device (device receives data) |
| transmission from the device |  |  | data will be send/transmitted from the device (device transmits data) |
| Telegram definition |  |  | Shows the telegram definition including abstract parameters as variables. |
| Example of the Telegram |  |  | Shows an example telegram with real values |
| optional parameter | [<px>] |  | Parameters placed in square brackets are optional. |
| device address | <d> | 0 … 999 | 0 = reserved for broadcast address 1…999 = device addresses |
| encoder number | <e> | 1 … 16 | 6 input channels: Enc 1 = In 1 Enc 3 = In 2 Enc 5 = In 3 Enc 7 = In 4 Enc 9 = In 5 Enc 11 = In 6 Enc 13 = Sys Ctrl Enc 15 = Out1 Enc 16 = Headphone 12 input channels: Enc 1 = In 1 Enc 2 = In 2 Enc 3 = In 3 Enc 4 = In 4 Enc 5 = In 5 Enc 6 = In 6 Enc 7 = In 7 Enc 8 = In 8 Enc 9 = In 9 Enc 10 = In 10 Enc 11 = In 11 Enc 12 = In 12 Enc 13 = Sys Ctrl Enc 14 = Out1 Enc 15 = Out2 Enc 16 = Headphone |
| encoder value | <ev> | 0 … 255 | encoder value/setting according to the actual system control selection |
| system control selection | <scs> |  | value (selection) of the system control encoder: 6 input channels: 1: = LEVEL 2: = TREBLE 3: = BASS 4: = LOW CUT 5: = LIMITER 6: = COMPRESSOR 7: = AUTOMIXING 8: = PRIORITY 9: = PAN / BALANCE 10: = Delay 11: = ROUTING TO OUT1 12: = ROUTING TO REC ,,, + Dig-Out Interface: 13: = ROUTING TO DIGOUT 14: = EQUALIZER 12 input channels: 1: = LEVEL 2: = TREBLE 3: = BASS 4: = LOW CUT 5: = LIMITER 6: = COMPRESSOR 7: = AUTOMIXING 8: = PRIORITY 9: = PAN / BALANCE 10: = Delay 11: = ROUTING TO OUT1 12: = ROUTING TO OUT2 13: = ROUTING TO REC …+ Dig-Out Interface: 14: = ROUTING TO DIGOUT 15: = EQUALIZER |
| Mute State | <mut> | 0, 1 | Mute state of the according encoder/channel: 0: encoder/channel not muted 1: encoder/channel muted |
| “ON”-LED State | <onl> | 0, 1 | “ON”-LED state of the according encoder: 0: LED is Off 1: LED is On |
| “PEAK”-LED State | <pkl> | 0, 1 | “PEAK”-LED state of the according encoder: 0: LED is Off 1: LED is On |
| channel number | <c> | 0 … 16 | 1…16 = automixer 6 input channels 1…16 = automixer 12 input channels |
| channel level | <cl> | 0 … 45 | Represents the input level at the according channel |
| channel status | <csx> | 00000 …45111 | Represents the input level, the ON and the Peak LED state of channel x in the following format: <cl><mut><onl><pkl> <cl>xxx : first two number represent <cl> xx<mutl>xx : third number represents <mut> xxx<onl>x : fourth number represents <onl> xxxx<pkl>: fifth number represents <pkl> |

## Version Commands

### Software Version („U2DM<d>SW?<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>SW?<CR> | This command requests the actual software version/info of the device. |
|  |  | U2DM1SW?<CR> |  |
|  |  | U2DM<d>SW=<p1>,<p2>,<p3><CR> | Device returns the software version: <p1>: SW-Version: (e.g.: 102 = 1.02) <p2>: Application Type 6: Digital Automixer <p3>: OEM Version (reserved) |
|  |  | U2DM1SW=121,6,0<CR> | <p1>: SW-Version: 1.21 <p2>: Application: Digital Automixer <p3>: OEM Version: 0 (reserved) |

### Hardware Version („U2DM<d>HW?<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>HW?<CR> | This command requests the actual hardware version/info of the device. |
|  |  | U2DM1HW?<CR> |  |
|  |  | U2DM<d>SW=<p1>,<p2>,<p3>,<p4>, <p5><CR> | Device returns the hardware version/info: <p1>: HW-Version <p2>: FPGA Version VHDL <p3>: FPGA Version MLAB <p4>: S-<card nr> (e.g.: 810 = S810) <p5>: Processor type (reserved) |
|  |  | U2DM1HW=1,1,2,830,0<CR> | <p1>: HW-Version = 1 <p2>: FPGA Version VHDL = 1 <p3>: FPGA Version MLAB = 2 <p4>: S<card nr> = S830 <p5>: Processor type = 0 (reserved) |

## Device Configuration

### Set Baudrate („U2DM<d>COM<c>BD=<bd>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>COM<c>BD=<bd><CR> | This command sets the baudrate of the specified COM port of the device: <c> = COM port 0: COM port actually connected to the device 1: COM 1 of the device 2: COM 2 of the device <bd>= Baudrate 24 (=2400 Baud) 48 (=4800 Baud) 96 (=9600 Baud) 192 (=19200 Baud) 384 (=38400 Baud) 576 (=57600 Baud) 1152 (=115200 Baud) Note: these settings can not be changed, they are fix: Parity: no Databits: 8 Stopbit(s): 1 |
|  |  | U2DM1COM0BD=1152<CR> | <p1>: Baudrate = 1152 (the new device baudrate is set to 115200 Baud) |
|  |  | --- | no device response |
|  |  | --- | no device response |

### Set Remote Interface („U2DM<d>DRI=<s>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>DRI=<s><CR> | This command enables or disables the remote interface of the device. For a remote control of the device, it is necessary to send this command first to enable the complete remote control functionality: <s>: 1 = enable the remote interface 0 = disable the remote interface |
|  |  | U2DM1DRI?<CR> | Request the state of the remote interface |
|  |  | U2DM1DRI=0<CR> | returns the state of the remote interface: <s>: 0 = remote interface disabled |
|  |  | U2DM1DRI=1<CR> | Enables the remote interface <s>: 1 = enables the interface |
|  |  | U2DM1DRI=1<CR> | Returns the state of the remote interface: <s>: 1 = remote interface is enabled |

## Encoder Commands

### Encoder Request Value („U2DM<d>EN<e>?<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>EN<e>?<CR> | This command requests the actual value of the selected encoder. <e>: selected encoder |
|  |  | U2DM1EN1?<CR> | Request the value of encoder 1 <e>: encoder nr = 1 |
|  |  | U2DM<d>EN<e>=<scs>,<ev><CR> | returns the settings of the selected encoder: <scs>: system control selection <ev>: encoder setting according to the actual system control selection <mut>: Mute State <onl>: “ON”-LED State <pkl>: “PEAK”-LED State |
|  |  | U2DM1EN1=1,26<CR> | returns the settings of the encoder 1: <scs>: System Control Setting Level (=1) <ev>: encoder value = 26 <mut>: encoder/channel not muted <onl>: “ON”-LED is Off <pkl>: “Peak”-LED is Off |

### Encoder Rotate Left („U2DM<d>EN<e>RL<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>EN<e>RL<CR> | This command executes a rotate left action at the selected encoder. <e>: selected encoder |
|  |  | U2DM1EN1RL<CR> | Executes a rotate left action at encoder 1 |
|  |  | U2DM<d>EN<e>=<scs>,<ev>,<mut>, <onl>,<pkl><CR> | returns the settings of the selected encoder: (see. “Encoder Request Value” command) |
|  |  | U2DM1EN1=1,26,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 26 |
|  |  | U2DM1EN1RL<CR> | Executes a rotate left action at encoder 1 |
|  |  | U2DM1EN1=1,27,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 27 |
|  |  | U2DM1EN1RL<CR> | Executes a rotate left action at encoder 1 |
|  |  | U2DM1EN1=1,28,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 28 |

### Encoder Rotate Right („U2DM<d>EN<e>RR<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>EN<e>RR<CR> | This command executes a rotate right action at the selected encoder. <e>: selected encoder |
|  |  | U2DM1EN1RR<CR> | Executes a rotate right action at encoder 1 |
|  |  | U2DM<d>EN<e>=<scs>,<ev>,<mut>, <onl>,<pkl><CR> | returns the settings of the selected encoder: (see. “Encoder Request Value” command) |
|  |  | U2DM1EN1=1,27,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 27 |
|  |  | U2DM1EN1RL<CR> | Executes a rotate left action at encoder 1 |
|  |  | U2DM1EN1=1,26,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 26 |
|  |  | U2DM1EN1RL<CR> | Executes a rotate left action at encoder 1 |
|  |  | U2DM1EN1=1,25,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 25 |

### Encoder Set Value („U2DM<d>EN<e>=<ev><CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>EN<e>=<ev><CR> | This command sets directly a new value to the encoder <e>: selected encoder <ev>: new encoder value |
|  |  | U2DM1EN1=32<CR> | sets encoder 1 to value of 32: <ev> = 32 |
|  |  | U2DM<d>EN<e>=<scs>,<ev>,<mut>, <onl>,<pkl><CR> | returns the settings of the selected encoder: (see. “Encoder Request Value” command) |
|  |  | U2DM1EN1=1,32,0,0,0<CR> | returns the settings of the encoder 1: <ev> = 32 |
|  |  | U2DM1EN13=3<CR> | sets encoder 13 (=system control) to value 3: <ev> = 3 (Bass) |
|  |  | U2DM1EN1=2,-4,0,0,0<CR> U2DM1EN2=2,0,0,0,0<CR> U2DM1EN3=2,0,0,0,0<CR> U2DM1EN4=2,0,0,0,0<CR> U2DM1EN5=2,0,0,0,0<CR> U2DM1EN6=2,0,0,0,0<CR> U2DM1EN7=2,2,0,0,0<CR> U2DM1EN8=2,0,0,0,0<CR> | returns the bass settings of all encoder/channels: |

## Status Commands

### Request Channel Status („U2DM<d>CS<c>?<CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>CS<c>?<CR> | This command requests the status of the selected channel from the device <c>: selected channel |
|  |  | U2DM1CS1?<CR> | Requests the channel status of channel 1 |
|  |  | U2DM<d>CS<c>=<cl>,<mut>,<onl>, <pkl><CR> | returns the status data of the selected channel <cl>: channel input level <mut>: Mute State <onl>: “ON”-LED State <pkl>: “PEAK”-LED State: |
|  |  | U2DM1CS1=6,0,1,0<CR> | Device returns the status of channel 1: <cl>: = 6 <mut>: = 0 <onl>: = 1 <pkl>: = 0 |

### Set Channel Status Remote Update Time („U2DM<d>CS<c>=<t><CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>CS<c>=<t><CR> | With this command you can set the time period (= t*100 ms) the device automatically sends a single channel status telegram for each channel: <c>: channel (has no effect) <t>: time period t*100 ms 0 = disables the automatic update 1 … 15 |
|  |  | U2DM1CS1=5<CR> | Set the automatically update period for the channel status us set to 5*100 ms (= 500 ms) |
|  |  | U2DM<d>CS<c>=<cl>,<mut>,<onl>, <pkl><CR> | returns the status of the selected channel: (see. “Request Channel Status” command) |
|  |  | U2DM1CS1=13,0,0,0<CR> <Delay t*100ms> U2DM1CS2=18,0,1,0<CR> <Delay t*100ms> U2DM1CS3=35,0,1,0<CR> <Delay t*100ms> U2DM1CS4=43,0,1,1<CR> <Delay t*100ms> U2DM1CS5=30,0,1,0<CR> <Delay t*100ms> U2DM1CS6=33,0,1,0<CR> <Delay t*100ms> U2DM1CS7=0,0,0,0<CR> <Delay t*100ms> U2DM1CS8=13,0,1,0<CR> <Delay t*100ms> U2DM1CS9=13,0,1,0<CR> <Delay t*100ms> ... | The device sends automatically the actual status of all channels. After telegram the device insertes a delay of t*100ms, in that example the delay is 500 ms. After transmitting the last channel the device starts again with channel number 1. Note: channel number 13 represents the system control, which status is always zero: “U2DM1CS13=0,0,0,0<CR>” |

### Channel Status Set Mute („U2DM<d>EN<c>MU=<mut><CR>“)

| Dir |  | Telegram | Description |
| --- | --- | --- | --- |
|  |  | U2DM<d>EN<e>MU=<mut><CR> | This command sets the mute state of the encoder <e>: selected encoder <mut>: Mute State |
|  |  | U2DM1EN1MU=1<CR> | Mutes encoder/channel 1 |
|  |  | U2DM<d>EN<e>=<scs>,<ev>,<mut>, <onl>,<pkl><CR> | returns the settings of the selected channel: (see. “Encoder Request Value” command) |
|  |  | U2DM1EN1=1,32,1,1,0<CR> | Device returns the setting of encoder 1: <mut>: encoder is muted |
|  |  | U2DM1EN1MU=0<CR> | Clears the mute state of encoder/channel 1 |
|  |  | U2DM1EN1=1,32,0,1,0<CR> | Device returns the setting of encoder1: <mut>: encoder is not muted |